ORION VLSI TECHNOLOGIES

Final Project

BMU Verification Plan

# Context

List Of Abbreviations	3

Introduction	4

Design Overview	6

Verification Strategy	8

Testbench Structure	12

Verification Extra Details	13

Open Items, Assumptions and Clarification Requests	14

'Open Items, Assumptions and Clarification Requests' is an addition beyond the original reference's section list. It is included because this plan must distinguish verified requirements from assumptions and unresolved questions explicitly, rather than leaving that distinction implicit in prose.

# List Of Abbreviations

| Abbreviation | Definition |
| --- | --- |
| BMU | Bit Manipulation Unit |
| UVM | Universal Verification Methodology |
| DUT | Design Under Test |
| RTL | Register Transfer Level |
| OP | Operation |
| RISC-V | Reduced Instruction Set Computer, Version 5 |
| Zbb | Basic Bit-Manipulation Subset |
| Zba | Address-Generation Bit-Manipulation Subset |
| Zbs | Single-Bit Manipulation Subset |
| Zbp | Permutation Bit-Manipulation Subset |
| SRL / SRA | Shift Right Logical / Arithmetic |
| SLL / ROL / ROR | Shift Left Logical / Rotate Left / Rotate Right |
| CLZ / CTZ | Count Leading Zeros / Count Trailing Zeros |
| CPOP | Count Population (set-bit count) |
| SLT / SLTU | Set-on-Less-Than (signed / unsigned) |
| MSB / LSB | Most / Least Significant Bit |
| CSR | Control and Status Register |
| VP | Verification Plan |
| TB | Testbench |
| DV | Design Verification |
| ISA | Instruction Set Architecture |
| ap | Decoded instruction control-signal struct (rtl_alu_pkt_t) |

# Introduction

## Goals of This Document

This document defines the verification plan for the Bit Manipulation Unit (BMU), a synthesizable RTL block compliant with the RISC-V BitManip extension (Zbb, Zbs, Zbp, and Zba subsets). It specifies the verification strategy, testbench architecture, the set of planned tests per operation, the coverage model, and the open items that must be resolved with the design/trainer before checker and coverage implementation can be considered complete. The plan is intended to be the single reference used to build, review, and sign off the BMU UVM environment.

## Scope of Verification

## The verification scope includes the top-level Bit_Manipulation_Unit and all associated logic contained within its design boundary.

## The BMU is verified as a standalone block through its documented top-level input and output interface.

## The functional scope includes the following 18 operations: OR, ORN, XOR, XNOR, SRL, SRA, ROR, BINV, SH2ADD, SUB, SLT, SLTU, CTZ, CPOP, SEXT.B, Signed MAX, PACK, and GREV/REV8.

## The scope also includes the documented CSR read and write functionality.

## The common BMU interface behavior associated with ap, operands, valid_in, reset, result_ff, and error is included.

## Verification is limited to normal functional operation using the approved parameter configuration with scan_mode = 0.

## Assumptions and Constraints

### Constraints

### The DUT RTL shall not be modified during the verification process.

### The BMU shall be verified as a standalone, black-box design.

### Verification shall be limited to the 18 approved BMU operations and the three documented CSR modes.

### scan_mode shall remain deasserted during functional verification.

### Undefined or insufficiently documented behavior shall not be assigned an expected result until clarified.

### Assumptions

- The design and verification environment use UVM — confirmed by the supplied architecture skeleton.

- The BMU specification, together with the approved design clarifications, provides the authoritative definition of the expected DUT behavior.

- The supplied RTL files, package definitions, parameters, and interface types represent the intended and mutually compatible project revision.

- The required simulation environment, UVM library, clock, and reset stimulus are available and correctly connected to the DUT.

# Design Overview

## The Bit Manipulation Unit (BMU) is a synthesizable, 32-bit RTL execution block intended to implement arithmetic, logical, shifting, counting, comparison, sign-extension, packing, and bit-manipulation functions associated with the RISC-V Bit Manipulation extensions.

## The BMU receives two 32-bit operands and a decoded control packet, ap, rather than a complete instruction encoding. Based on these inputs, it selects the requested operation, generates the corresponding result, and detects invalid or conflicting control combinations. The block also supports CSR read and write data-selection paths.

## The computed result is captured in the registered output result_ff. For a request accepted with valid_in = 1 in cycle N, the corresponding result is available in cycle N+1. When valid_in = 0, result_ff retains its previous value.

## Key Features

- Compliant with the RISC-V Bit Manipulation extensions listed below.

- Integrated control and data path.

- Provides compile-time feature enablement through the pt.BITMANIP_* parameter set.

- Robust error-detection logic: mechanisms to detect and respond to invalid operations or unexpected control-field conditions.

- Produces a registered 32-bit result with one-cycle latency.

## Supported Extensions

## The following table lists the operations identified in the specification’s Features section. This list describes the advertised design capabilities and does not define the current verification scope. The operations selected for verification are identified separately in the Scope of Verification section.

| Extension | Operations (as named in the specification) |
| --- | --- |
| Zbb | CLZ, CTZ, CPOP, MIN, MAX, ANDN, ORN, XNOR, SEXT.B, SEXT.H |
| Zbs | BSET, BCLR, BINV, BEXT |
| Zbp | ROL, ROR, PACK, PACKU, PACKH, GREV (rev8 subset), GORC (orc_b subset) |
| Zba | SH1ADD, SH2ADD, SH3ADD |

Each instruction is conditionally enabled based on its corresponding pt.BITMANIP_XXX compile-time parameter, which is distinct from the runtime ap.zbb-style qualifier bits used within individual operations.

## Block Diagram and Interface

The interface and signal details are listed below.

| Signal | Width | Direction | Description |
| --- | --- | --- | --- |
| clk | 1 bit | Input | Clock signal to the module |
| rst_l | 1 bit | Input | Active-low synchronous reset; resets all internal registers to their initial state |
| a_in | 32 bits | Input | First operand of the BMU module |
| b_in | 32 bits | Input | Second operand of the BMU module |
| scan_mode | 1 bit | Input | Scan test-mode control , held at 0 during functional verification |
| valid_in | 1 bit | Input | Instruction valid flag — indicates whether the input instruction is valid |
| ap | Struct | Input | Decoded instruction control signals (rtl_alu_pkt_t in the RTL/testbench) |
| csr_ren_in | 1 bit | Input | CSR read-enable |
| csr_rddata_in | 32 bits | Input | CSR read data |
| result_ff | 32 bits | Output | Final computed result |
| error | 1 bit | Output | Error indicator |

# Verification Strategy

## Methodology

SystemVerilog and UVM shall be used to verify the BMU as a standalone DUT through directed and constrained-random simulation.

## The testbench shall include a sequence item, sequences, sequencer, driver, monitor, agent, scoreboard, functional-coverage collector, environment, tests, interface, and top-level testbench module.

## An independent reference model shall be implemented in SystemVerilog and used by the scoreboard to predict the expected BMU results based on the specification and approved design clarifications.

## Selection of Verification Tests

The approved verification baseline covers 18 documented BMU operations and three CSR data-selection modes.

The covered functional groups are:

- Logical Operations

  - OR

  - ORN

  - XOR

  - XNOR

- Shift, Rotate, and Bit-Index Operations

  - SRL

  - SRA

  - ROR

  - BINV

- Arithmetic and Comparison Operations

  - SH2ADD

  - SUB

  - SLT

  - SLTU

  - Signed MAX

- Counting and Sign-Extension Operations

  - CTZ

  - CPOP

  - SEXT.B

- Packing and Generalized-Reverse Operations

  - PACK

  - GREV with the documented REV8 mode

- CSR Data-Selection Operations

  - CSR read

  - CSR write using operand A

  - CSR write using operand B

Additional tests cover synchronous reset, valid_in behavior, result retention, legal operation transitions, invalid and conflicting ap combinations, unsupported GREV mode selections, and recovery from an error-producing request.

## The companion workbook, BMU_Sequence_Test_Plan_Final.xlsx, is the primary test-tracking document for this verification effort.

## Specific Corner Cases

The verification tests include the following operation-specific and interface-level corner cases:

- Logical Operations

  - Zero and all-one operands.

  - Equal and complementary operands.

  - Alternating-bit patterns.

  - Complete per-bit logical truth tables.

  - Transitions between the base and qualified operation modes.

- Shift and Rotate Operations

  - All shift amounts from 0 through 31.

  - Positive and negative operand patterns.

  - One-hot and walking-bit operands.

  - Verification that upper unused bits of b_in do not affect the result.

- Bit Inversion

  - Every selectable bit position from 0 through 31.

  - Original selected-bit values of both zero and one.

  - Verification that exactly one selected bit is inverted.

- Arithmetic Operations

  - Zero operands and equality cases.

  - Carry and borrow propagation.

  - Modulo-232 wraparound.

  - Maximum positive, minimum negative, and boundary operand values.

  - SH2ADD behavior when significant bits are shifted out.

- Comparison Operations

  - Less-than, equal, and greater-than relations.

  - All operand-sign combinations.

  - Signed and unsigned interpretation boundaries.

  - Cases where signed and unsigned comparisons produce different results.

- Counting Operations

  - CTZ results from 0 through 32.

  - Explicit verification that CTZ(0) returns 32.

  - CPOP results from 0 through 32.

  - Walking-one, walking-zero, alternating, zero, and all-one patterns.

- Sign Extension

  - Exhaustive testing of all 256 possible low-byte values.

  - Positive and negative sign-extension cases.

  - Verification that upper input bits do not affect the result.

- PACK

  - Independent propagation of every bit from both source halfwords.

  - Asymmetric operand patterns to detect source-order reversal.

  - Verification that the unused upper halves do not affect the result.

- GREV/REV8

  - The documented legal REV8 mode.

  - One-hot byte-reversal patterns.

  - Unsupported mode-select values.

  - Verification of the documented zero-result and error behavior for unsupported selections.

- CSR Paths

  - CSR read-data selection.

  - CSR write selection from operand A.

  - CSR write selection from operand B.

  - Zero, all-one, one-hot, and structured data patterns.

  - Reviewed CSR and BMU-control conflicts.

- Reset and valid_in

  - Reset at startup and during active traffic.

  - Reset with valid_in equal to both zero and one.

  - Result update when valid_in = 1.

  - Result retention for short and extended valid_in = 0 intervals.

  - Consecutive accepted operations without idle cycles.

- Error and Recovery

  - Missing required qualifiers.

  - Forbidden or conflicting ap fields.

  - Multiple operation selections.

  - CSR and BMU-operation conflicts.

  - Unsupported operand encodings.

  - Legal operation following an error-producing request.

  - Error behavior with both values of valid_in.

## Coverage Targets

### Code Coverage

The following code-coverage objectives apply to the BMU DUT hierarchy:

- Statement/line coverage: at least 95%.

- Branch coverage: at least 95%.

- Condition/expression coverage: at least 90%.

- Toggle coverage: at least 90%.

- All remaining uncovered items must be reviewed and classified as reachable, unreachable, disabled by configuration, or outside the approved scope.

- Dead or redundant RTL must not be accepted solely because the overall percentage target has been reached.

No functional FSM-coverage target is defined because the available BMU specification does not identify a standalone functional FSM. Any simulator-reported internal state coverage will be reviewed as part of code coverage.

### Functional Coverage

The target is 100% closure of all approved and reachable functional-coverage bins.

Functional coverage includes:

- All 21 approved operation and CSR mode identifiers.

- Operand-A classes for every documented operation.

- Operand-B classes for operations in which b_in is a data operand.

- Logical truth-table combinations.

- Shift and rotate amounts from 0 through 31.

- BINV bit indices and original bit values.

- Arithmetic carry, borrow, relation, and wraparound cases.

- Signed and unsigned comparison relations.

- CTZ and CPOP result ranges.

- Exhaustive SEXT.B low-byte values.

- PACK source-lane and bit-propagation coverage.

- GREV legal and unsupported mode classes.

- CSR source-selection and bit-propagation coverage.

- Confirmed illegal control combinations.

- Request class × valid_in × observed error.

- Reset × legal mode × valid_in.

- valid_in transitions and result-hold lengths.

- All ordered legal-mode transitions.

- Error-recovery sequences following confirmed conflicts.

Cross coverage is limited to meaningful and reachable combinations. Impossible or specification-disabled combinations will be excluded only after review and documented justification.

## Test and Result Tracking

The companion workbook, BMU_Sequence_Test_Plan_Final.xlsx, is the primary test-tracking document for this verification effort.

It contains:

- 25 sequence/test pairs.

- 133 uniquely identified verification scenarios.

- Operation-specific stimulus and expected behavior.

- Coverage IDs linked to every scenario.

- Execution gates and priorities.

- Status, test date, and actual-result fields.

- Evidence, issue, and responsibility notes.

- Functional-coverage and cross-coverage registers.

- Run-log and open-decision tracking.

A scenario may be marked Passed only after all mandatory scoreboard comparisons, interface checks, and resolved error checks have completed successfully. Coverage percentages must be entered only from an actual simulator-generated coverage report.

At the time of planning, no simulation results have been supplied. Therefore, the workbook contains no claimed passing tests or measured coverage results. Cycle-accurate error checking remains blocked until the timing of the error output is confirmed.

# Testbench Structure

Testbench Components

The BMU verification environment consists of the following UVM components:

- Sequence Item: Represents one BMU transaction and contains the DUT input controls, operands, and monitored outputs required for checking and coverage.

- Sequencer: Supplies sequence items generated by the active sequence to the driver.

- Driver: Converts sequence items into pin-level stimulus and drives the BMU input signals through the virtual interface.

- Monitor: Samples the DUT inputs and outputs without driving them, constructs observed transactions, and publishes them to the scoreboard and coverage collector.

- Agent: Groups the sequencer, driver, and monitor and connects the sequencer to the driver.

- Reference Model: Independently calculates the expected BMU result and expected control behavior for each supported request.

- Scoreboard: Uses the reference-model prediction to compare the expected response with the monitored result_ff and error, while accounting for reset, valid_in, and the registered result latency.

- Coverage Collector: Samples monitored transactions and measures operation, operand, corner-case, control-legality, timing, transition, error, reset, and recovery coverage.

- Environment: Instantiates and connects the agent, scoreboard, and coverage collector.

- Sequences and Tests: Generate the directed, constrained-random, negative, reset, CSR, timing, and mixed-operation scenarios defined in the test-plan workbook.

Testbench Interfaces and Connections

The SystemVerilog interface bmu_if provides the pin-level connection between the UVM testbench and the BMU DUT. It contains the following DUT signals:

- clk

- rst_l

- scan_mode

- valid_in

- a_in

- b_in

- ap

- csr_ren_in

- csr_rddata_in

- result_ff

- error

The interface defines two clocking blocks:

- cb_drv, synchronized to the negative edge of clk, is used by the driver to apply DUT inputs.

- cb_mon, synchronized to the positive edge of clk, is used by the monitor to sample DUT inputs and outputs.

The interface also declares DRV and MON modports. However, the current driver and monitor obtain a handle of type virtual bmu_if and access the corresponding clocking blocks directly.

The top-level module, bmu_tb_top.sv, creates the physical interface instance and passes its virtual handle to the agent components through uvm_config_db.

Internal UVM communication uses transaction-level connections:

- The sequencer transfers sequence items to the driver through the standard sequencer-driver connection.

- The monitor publishes observed transactions through its analysis port.

- The environment connects the monitor analysis port to the scoreboard and coverage subscriber.

These UVM transaction-level connections are separate from the signal-level bmu_if connection between the testbench and DUT.

The driver and monitor access this interface through virtual-interface handles. The driver uses the cb_drv clocking block to drive DUT inputs on the negative clock edge, while the monitor uses the cb_mon clocking block to sample the

# Verification Extra Details

## Tools and Technologies

| Tool | Intended Use |
| --- | --- |
| Cadence Xcelium (xrun) | SystemVerilog/UVM compilation, elaboration, simulation, assertions, and coverage generation. |
| Integrated Metrics Center (IMC) | Functional and code-coverage analysis, merging, reporting, and coverage-hole review. |
| Cadence Verisium Debug | Waveform inspection and simulation-debug support, when available in the training environment. |
| MobaXterm | Remote access to the training server and terminal environment. |
| Visual Studio Code | Testbench development, source review, and project organization. |
| Git and GitHub | Source control, change tracking, and project delivery. |

## Regression Execution

The regression will execute the implemented UVM tests as separate simulation runs. Each run will select one test using +UVM_TESTNAME, and the regression script will collect the resulting logs, pass/fail status, seeds, and coverage databases.

The tests will not all be started simultaneously inside one bmu_tb_top simulation. Separate runs provide failure isolation, reproducible random seeds, clearer logs, and reliable coverage merging.

The current top-level module calls:

run_test("bmu_smoke_test")

Before regression execution, this should be changed to:

run_test()

This allows the required test to be selected from the command line through +UVM_TESTNAME.

## Regression Tests

A regression test set is planned to run every implemented directed, error, and constrained-random test together in the top-level testbench module, rather than running each test individually. Regression tiers:

| Tier | When | Included Work |
| --- | --- | --- |
| Bring-up | Before any closure claim | Compilation, reference-model unit checks, interface connectivity, reset release, smoke transactions, and initial scoreboard comparisons. |
| Directed baseline | After the oracle/timing model and scope questions are resolved | All directed per-operation and CSR scenarios, approved negative tests, and reset/hold/transition tests. |
| Extended Regression | After the directed baseline passes consistently | Constrained-random campaigns, error and recovery scenarios, multiple seeds, risk-focused scenarios, and preserved failing seeds. |
| Coverage Closure | After functional regressions are stable | Targeted tests for uncovered functional bins, cross-coverage holes, assertion coverage, and reviewed code-coverage gaps. |
| Sign-off | After changes and coverage-hole review | Full mandatory suite plus targeted closure campaigns and confirmation seeds. |

## Coverage Reports

Will be attached later, once the environment is built and an initial regression has been run against the DUT.

## Verification Results

Will be attached later, once regression and coverage-closure activities are complete. No results currently exist.

# Open Items, Assumptions and Clarification Requests

This section consolidates every unresolved item referenced elsewhere in this plan into a single, prioritized list. Items 1 and 2 are the highest priority — they block compilation and expected-value checking regardless of which operation is being tested. This list is the single reference to route to the trainer for sign-off; once an item is resolved, update the corresponding section of this plan and remove the row rather than leaving it marked open.

| # | Missing Information | Why Current Sources Don't Resolve It | What Depends on the Answer | Status |
| --- | --- | --- | --- | --- |
| 1 | Final operation scope for the 16 deferred operations (AND, ANDN, SLL, ROL, BSET, BCLR, BEXT, SH1ADD, SH3ADD, ADD, CLZ, SEXT.H, MIN, PACKU, PACKH, GORC) | Features table commits to support; no section documents control fields, guard conditions, or error behavior. | Scope of Verification, Selection of Verification Tests, and all associated coverage bins. | Open — largest volume of blocked work |
| 2 | rtl_alu_pkt_t field-name confirmation | The ap struct's actual field declaration lives in rtl_pkg, which is not included in the supplied skeleton; every ap.* name used in this plan is drawn from the specification, not a confirmed RTL declaration. | Every constrained/directed stimulus assignment; compilation of any operation-specific sequence. | Open — blocks all sequence implementation |
| 3 | Error/result timing and cycle alignment (relative to reset and driven inputs) | The skeleton's own scoreboard code defers prediction logic pending this; the working oracle assumption (Introduction) is a reasonable hypothesis but is unconfirmed. | Reference model implementation; every expected-error check. | Open — blocks scoreboard |
| 4 | OR/XOR-family error-value contradiction | The specification's own summary table and worked Example 3 disagree for the identical stimulus condition (conflicting fields, csr_ren_in=0). | Expected error value for at least six operations' negative tests. | Open |
| 5 | CSR overlap and orphan-qualifier behavior (simultaneous read+write; csr_imm asserted without csr_write) | No documented rule covers these combinations; the confirmed CSR baseline (Read, Write-A, Write-B) does not address them. | Two CSR negative-test scenarios. | Open |
| 6 | valid_in=1 with ap=0 and csr_ren_in=0 (no operation selected) | Never documented; no worked example exercises it. | One negative-test scenario and its coverage bin. | Open |
| 7 | MAX / MIN unsigned mode | SLT documents ap.unsign; MAX's section never mentions it, and MIN has no documented section at all. | MAX coverage scope; MIN test cases once Item 1 is resolved. | Open |
| 8 | RTL file list — README.md vs. sim/filelist.f disagree on which RTL source files exist under rtl/ | Two project documents name different files (README also lists library/rtl_param.vh, which filelist.f omits; filelist.f adds rtl_pdef.sv and rtl_defines.sv, which README never mentions). | First compile attempt on the Orion server. | Open — blocks compile |
| 9 | scan_mode normal (non-scan) operating value | bmu_driver.sv carries this as an explicit placeholder pending trainer confirmation. | Precondition state for every directed test. | Open |

## Document Lineage and Traceability

This plan was produced by reconciling five sources: the original BMU Verification Plan draft (structural reference for section organization), an earlier professional revision of the same plan, the BMU Specification Document v1.1, the supplied BMU UVM Architecture Skeleton (read directly, file by file), and the BMU Detailed Test Plan. Where the Detailed Test Plan's own source list cites a "BMU_Verification_Plan_Professional.docx, review draft v0.3" and an "Orion Verification Planning guide" PDF, neither document was supplied for this consolidation — content attributed to them in the Detailed Test Plan has not been independently verified here and is not claimed as confirmed in this plan.

No content in this document represents an executed test, a measured coverage result, or observed DUT behavior. Every operation's expected-value behavior is derived from the specification text and its worked examples; every architecture status is derived from direct inspection of the supplied skeleton code; every item in this section is an open question, not an assumed answer.